# Solar Power Forecasting

Adarsh Chauhan

230107008

Final Project submission

Course Name: Applications of AI and ML for Chemical Engineering

Course Code: CL653

Submission Date: April 23, 2026

Contents

1	Executive Summary	3

2	Introduction	3

3	Methodology	4

4	Implementation Plan	9

5	Testing and Deployment	11

6	Results and Discussion	13

7	Conclusion and Future Work	17

8	References	19

9	Appendices	20

10	Auxiliaries	22

# Executive Summary

This project develops a comprehensive Solar Power Generation Forecasting System that combines classical Machine Learning, Seasonal Time-Series modelling (SARIMA), and Deep Learning (LSTM) to accurately predict solar AC power output. Built on real operational data from two solar plants in India (Plant 1 and Plant 2), the system addresses one of the most pressing challenges in renewable energy infrastructure: the inherent unpredictability of solar generation due to weather variability.

The dataset spans 34 days of 15-minute interval measurements from two plants with 22 inverters each. The pipeline engineers 14+ features with explicit leakage avoidance — specifically, EFFICIENCY (which uses the target variable) and DAILY_YIELD (a cumulative within-day counter) are excluded. Five model classes are trained: Linear Regression, Decision Tree, Random Forest with GridSearchCV, SARIMA, and LSTM.

SARIMA is applied at two granularities: hourly plant-level aggregates (~816 data points, seasonal period s=24) as the primary model enabling full four-panel residual diagnostics, and daily aggregates (34 points, s=7) as an alternative with two-panel simplified residual analysis. The Random Forest achieves R² > 96% on held-out 15-minute data. A Streamlit web application, deployable in two commands with seven Python packages, offers an interactive hourly/daily SARIMA toggle accessible to engineers without ML expertise.

# Introduction

Background: Solar power forecasting was selected for this project for three intersecting reasons: scientific richness, real-world urgency, and direct relevance to Chemical Engineering.

Scientifically, the problem sits at the intersection of time-series modelling, environmental physics, and machine learning. It requires understanding both the mathematical structure of sequential data and the domain physics — how irradiance, angle of incidence, module temperature, and atmospheric scattering interact to drive inverter output. No single model class dominates; the problem genuinely benefits from the full spectrum from classical statistics (SARIMA) to ensemble ML (Random Forest) to deep learning (LSTM), making it ideal for demonstrating AI/ML breadth in a single coherent project.

From an urgency standpoint, India is among the world's fastest-growing solar markets, targeting 500 GW of renewable capacity by 2030. The dataset used originates from a real Indian operational plant, making findings directly applicable. Forecasting errors translate to grid instability and financial penalties under India's Deviation Settlement Mechanism (DSM), where generators pay ₹3–12 per unit for deviations from declared schedules.

For this course specifically, Chemical Engineering and Process Industries are among the largest industrial electricity consumers. Accurate solar forecasting directly impacts batch scheduling, reactor temperature control, distillation reboiler management, and energy procurement strategy — all core Chemical Engineering concerns.

Problem Statement: Traditional solar forecasting methods — persistence models, climatological averages, and physics-only NWP simulations — share a fundamental limitation: they cannot model the complex, non-linear, temporally-dependent interactions between weather variables and actual inverter output. They also fail to adapt to plant-specific characteristics, inverter degradation, or short-term weather transients.

This project addresses four specific gaps:

- Short-term (15-minute to hourly) solar AC power prediction at both inverter level and plant level with quantified uncertainty

- Multi-model comparison: classical statistics (SARIMA) vs ensemble ML (Random Forest) vs deep learning (LSTM)

- Automated fault and anomaly detection from generation patterns, without additional sensor infrastructure

- Accessible deployment for non-ML-expert engineers through a web interface requiring no specialist software or SCADA integration

# Methodology

Data Source:

Dataset: Kaggle — Solar Power Generation Data (public domain). URL: https://www.kaggle.com/datasets/anikannal/solar-power-generation-data

| File | Contents | Resolution |
| --- | --- | --- |
| Plant_X_Generation_Data.csv | DC_POWER, AC_POWER, DAILY_YIELD, TOTAL_YIELD per inverter (22 inverters per plant) | Every 15 minutes |
| Plant_X_Weather_Sensor_Data.csv | AMBIENT_TEMPERATURE, MODULE_TEMPERATURE, IRRADIATION (plant-level sensor) | Every 15 minutes |

Two plants are supported via a PLANT_ID = 1 or 2 toggle. The generation file contains one row per (timestamp, inverter); the weather file contains one row per timestamp. Merge is performed on DATE_TIME after deduplicating the weather file.

Data Preprocessing:

- Datetime parsing with auto-format detection (dayfirst=True) for both CSV date formats

- Merge on DATE_TIME; weather CSV deduplicated before merge to prevent row multiplication

- Physical validity filter: rows where AC_POWER < 0 or DC_POWER < 0 are removed (sensor fault indicator)

- Null removal on five critical columns: AC_POWER, DC_POWER, IRRADIATION, AMBIENT_TEMPERATURE, MODULE_TEMPERATURE

- Deduplication on (DATE_TIME, SOURCE_KEY) pairs to remove any duplicate inverter records

Model Architecture:

ML Models — Linear Regression, Decision Tree, Random Forest

All three ML models are trained on 15-minute per-inverter data using an 80/20 temporal split (shuffle=False to preserve time order). The Random Forest uses GridSearchCV with TimeSeriesSplit(n_splits=5) — prevents future data appearing in training folds during hyperparameter search.

| Model | Configuration | Key Characteristics |
| --- | --- | --- |
| Linear Regression | StandardScaler + OLS; all leakage-free features | Interpretable baseline; assumes linear relationships; fastest to train |
| Decision Tree | max_depth=15, min_samples_split=10 | Captures non-linearity; no scaling required; prone to overfit at high depth |
| Random Forest ★ | n_estimators:[100,200]; max_depth:[10,20,None]; min_samples_split:[5,10]; max_features:[sqrt,log2]; TimeSeriesSplit(5) | Best tabular accuracy; feature importance; robust to outliers; recommended production model |

### SARIMA — Seasonal Time-Series Model

SARIMA (Seasonal AutoRegressive Integrated Moving Average) is the standard classical approach for time series with periodic structure. Solar generation has a strong daily seasonal pattern — the bell-curve shape peaking at solar noon — making SARIMA a principled choice. SARIMA(p,d,q)(P,D,Q,s) notation:

- p=1, d=1, q=1: AR(1) + first-order differencing for stationarity + MA(1) for error correction

- P=1, D=1, Q=0: seasonal AR(1) + seasonal differencing to remove periodic mean; Q=0 (no seasonal MA needed)

- s: seasonal period — number of time steps in one full cycle (24 for hourly-daily; 7 for daily-weekly)

| Config | Granularity | Points | Period s | Train pts | Residual Analysis Level |
| --- | --- | --- | --- | --- | --- |
| SARIMA(1,1,1)(1,1,0,24) ★ Primary | Hourly plant totals | ∼816 | 24 (daily cycle) | ∼653 | Full 4-panel diagnostics (all valid) |
| SARIMA(1,1,1)(0,0,0,0) ◦ Alternative | Daily plant totals | 34 | 7 (weekly cycle) | 27 | 2-panel residual analysis only |

Understanding Residual Analysis vs Full Diagnostics:

Both SARIMA configurations perform residual analysis — examining differences between model-fitted values and actual observations. The two levels differ in which statistical tests can be reliably applied at each sample size:

| Analysis Component | Daily SARIMA (n–27) | Hourly SARIMA (n≈653) | Reason for difference |
| --- | --- | --- | --- |
| Residuals over time (bar plot) | Yes ✔ | Yes ✔ (panel 1 of diagnostics) | Always valid; shows if errors have temporal patterns |
| Residual distribution (histogram) | Yes ✔ | Yes ✔ (panel 2 of diagnostics) | Always valid; checks approximate normality |
| Normal Q-Q plot | No ✘ (unreliable) | Yes ✔ (panel 3 of diagnostics) | Requires 40+ points; tails are meaningless at n=27 |
| ACF of residuals (correlogram) | No ✘ (unreliable) | Yes ✔ (panel 4 of diagnostics) | Confidence bands too wide at n=27; 50+ needed |
| Ljung-Box formal test | No ✘ (insufficient df) | Readable from ACF panel | Formal white-noise test needs sufficient degrees of freedom |

The statsmodels plot_diagnostics() function always produces all four panels. For the daily SARIMA, panels 3 and 4 are computed but flagged as unreliable and not interpreted. For the hourly SARIMA, all four panels are valid and interpreted fully.

### LSTM — Deep Learning Extension

Two-layer LSTM: LSTM(128, return_sequences=True) → Dropout(0.2) → BatchNorm → LSTM(64) → Dropout(0.2) → BatchNorm → Dense(32, ReLU) → Dense(1). Sequence length = 24 steps (~6 hours of 15-min data). Huber loss is used for robustness to zero-power nighttime outliers.

Original training used 50 epochs, which was identified as excessive: val_loss plateaus around 10–15 epochs, meaning the additional 35–40 epochs waste compute without improving accuracy. This was corrected to a maximum of 20 epochs with EarlyStopping(patience=5), reducing wall-clock training time approximately 3× while achieving equivalent or better accuracy through the best-weights restore mechanism.

Tools and Technologies:

| Tool / Library | Version | Purpose |
| --- | --- | --- |
| Python | 3.10+ | Core programming language |
| Pandas / NumPy | 2.0 / 1.24 | Data manipulation and numerical computing |
| Scikit-learn | 1.3+ | ML models, GridSearchCV, TimeSeriesSplit, metrics, permutation importance |
| Statsmodels | 0.14+ | SARIMA (SARIMAX), plot_diagnostics(), seasonal decomposition |
| TensorFlow / Keras | 2.13+ | LSTM deep learning model (notebook only; not in Streamlit app) |
| Matplotlib / Seaborn | 3.7 / 0.12 | EDA dashboards, model evaluation plots, day-wise profiles |
| Streamlit | 1.32+ | Web application with hourly/daily SARIMA toggle (7-package install total) |
| Joblib | 1.3+ | Model persistence — RF .pkl, LSTM scalers .pkl |

# Implementation Plan

Development Phases:

| Phase | Task | Status |
| --- | --- | --- |
| Phase 1 | Project proposal, motivation analysis, problem definition, literature review, methodology design | Completed |
| Phase 2 | Data pipeline: loading, merging, cleaning, feature engineering with leakage avoidance (EFFICIENCY and DAILY_YIELD excluded) | Completed |
| Phase 3 | EDA: Spearman correlation, distributions, hourly AC profile, fault detection (CoV), weather trends, seasonal decomposition | Completed |
| Phase 4 | ML models: LR, DT, RF+GridSearchCV + Gini importance + Permutation importance (10 repeats) | Completed |
| Phase 5 | SARIMA hourly (primary, s=24, full diagnostics) + daily (alternative, s=7, 2-panel residual); LSTM extension (20 epochs) | Completed |
| Phase 6 | Streamlit app: hourly/daily toggle, interactive EDA dashboard, comparison tab, JSON report download | Completed |
| Phase 7 | Report: real-life applications, challenges, advantages vs existing systems, consistency review | Completed |

Model Training:

- ML 80/20 temporal split (shuffle=False): last 20% of rows are the test set; no test rows seen during training or GridSearchCV hyperparameter search

- TimeSeriesSplit(n_splits=5): each fold uses only past data to validate future data; prevents temporal leakage during cross-validation

- SARIMA hourly: 80/20 temporal split (≈653 training hours, ≈163 test hours); forecast horizon = 2+ full seasonal cycles

- SARIMA daily: 27 training days, 7 test days; forecast horizon = 1 seasonal cycle

- LSTM: max 20 epochs, EarlyStopping(patience=5), ReduceLROnPlateau(factor=0.5, patience=4); MinMaxScaler applied to both features and target separately for correct inverse transformation

Model Evaluation:

| Metric | Formula | Notes |
| --- | --- | --- |
| MAE | mean(¦actual−predicted¦) | Average absolute error in kW; intuitive physical unit; reported per-model |
| RMSE | √mean((actual−predicted)²) | Penalises large errors more than MAE; primary model selection criterion |
| R² | 1−SS_res/SS_tot | Fraction of variance explained; scale-independent; used as cross-model comparison indicator |
| MAPE (daytime only) | mean(¦actual−pred¦/actual)×100 where AC_POWER>0 | Computed on daytime rows only; nighttime near-zero values excluded to avoid division artefact |

# Testing and Deployment

Testing Strategy:

| Issue | Root Cause | Fix Applied |
| --- | --- | --- |
| RF predict on X_train (bug) | Copy-paste error; training set used for evaluation; artificially inflated metrics | Explicit rf.predict(X_test) with comment; held-out test set only |
| EFFICIENCY feature leakage | EFFICIENCY = AC_POWER/(IRRAD×1000) directly uses the target variable | Excluded from all ML and LSTM feature sets; reason documented in code and report |
| DAILY_YIELD feature leakage | Cumulative within-day counter encodes same-day future energy readings | Excluded; autoregressive lag features (t-1, t-3) provide safe historical signal |
| SARIMA memory on 15-min data | Fitting on 68,000 high-frequency rows causes memory overflow | Hourly aggregation: ~816 plant-level points; also improves seasonal signal quality |
| SARIMA diagnostics unreliable on daily data | 27 training points; ACF and Q-Q confidence bands are too wide for inference | Hourly SARIMA (653 training pts) used as primary with full diagnostics; daily shows 2-panel residual analysis |
| ARIMA vs SARIMA inconsistency | Earlier versions used ARIMA in report but SARIMAX in code | SARIMA used consistently in all three artefacts; full audit completed |
| LSTM 50 epochs — excessive | val_loss plateaus at ∼10–15 epochs; extra epochs waste compute with no benefit | Max 20 epochs + EarlyStopping(patience=5); best weights restored automatically |
| MAPE inflation by nighttime zeros | AC_POWER≈0 at night; division produces arbitrarily large values | MAPE computed on daytime rows only (AC_POWER>0); documented in all metric outputs |

Deployment Strategy:

| Option | Steps | Cost |
| --- | --- | --- |
| Local (development) | pip install -r requirements.txt && streamlit run app.py (opens at http://localhost:8501) | Free |
| Streamlit Community Cloud | Push to GitHub → connect at share.streamlit.io → click Deploy (∼2 minutes) | Free |
| Cloud VM (AWS/GCP/Azure) | Install requirements, open port 8501, run streamlit run app.py as background process | VM cost only (∼₹500/month for smallest instance) |

Requirements: 10 packages (streamlit, pandas, numpy, matplotlib, seaborn, scikit-learn, statsmodels, joblib, jinja2, scipy). No GPU, no SCADA, no specialised hardware. Minimum Python 3.10. Install size ~120 MB. TensorFlow (for LSTM) is a notebook-only dependency; not required for the Streamlit app.

Ethical Considerations:

- Public Kaggle dataset; no personal, private, or commercially sensitive data processed

- System provides decision support only — no autonomous grid control, dispatch actions, or safety-critical decisions

- Model limitations (sample size, SARIMA diagnostic validity, scale mismatch) communicated clearly in the user interface through callout boxes

- 95% confidence intervals shown explicitly so grid operators are not misled by overconfident point forecasts

- Feature leakage decisions fully documented with root cause; all exclusions explained in code comments and report

# Results and Discussion

Findings:

| Observation | Implication for Modelling |
| --- | --- |
| Irradiation: highest Spearman ρ with AC_POWER (~0.90+) | Primary physical driver; top RF feature; motivates SARIMA seasonal structure (bell-curve peak) |
| Clear bell-curve hourly profile — peak at 10:00–15:00 | Motivates SARIMA seasonal period s=24 (daily cycle); TOTAL_MINUTES_PASSED outperforms discrete HOUR for ML |
| Module temperature 5–20 °C above ambient throughout generation hours | DELTA_TEMPERATURE carries real thermodynamic signal; confirms heat transfer physics in data |
| CoV > 0.70 on 4+ days | Flags cloud-cover events and potential inverter fault days; actionable maintenance alerts |
| DC/AC ratio ≈93–95% during generation hours | Consistent with high-quality inverter efficiency specs; confirms data integrity and valid sensor readings |
| DC_POWER and AC_POWER Spearman ρ > 0.99 | Near-perfect correlation; DC_POWER is a valid and strong predictor for AC_POWER without being the target itself |

Comparative Analysis:

| Model | MAE (kW) | RMSE (kW) | R² (%) | MAPE* (%) | Notes |
| --- | --- | --- | --- | --- | --- |
| Linear Regression | 63.941 | 113.400 | 91.76 | 34.24 | Interpretable baseline; linear assumption limiting |
| Decision Tree | 21.461 | 62.024 | 97.54 | 6.21 | Captures non-linearity; overfits without depth control |
| Random Forest ★ (tuned) | 13.104 | 40.532 | 98.95 | 3.95 | Recommended production model; best ML accuracy |
| LSTM | 36.441 | 76.946 | 96.21 | 21.60 | Temporal; captures ramp patterns; slower to train |

* MAPE on daytime rows only (AC_POWER > 0). Metric ranges are indicative across both plants; exact values depend on run and plant.

| Reference Approach | Approach Used | This System's Advantage |
| --- | --- | --- |
| Persistence + NWP (industry standard) | Physics-based atmospheric model | ML captures plant-specific non-linearities that NWP misses; no meteorological data service required |
| Single LSTM (common academic) | One LSTM on raw data, single plant | Multi-model comparison; feature engineering substantially improves accuracy; two plants; deployment-ready |
| Prophet (Facebook/Meta) | Additive trend + seasonal decomposition | SARIMA is more principled for short regular series; direct CI from posterior; no black-box components |
| Commercial SCADA fault detection | Fixed-threshold rule-based alerts | CoV-based detection is statistically adaptive to plant baseline; no threshold calibration needed |
| Single-model academic papers | Typically one model, one dataset, no deployment | Five models compared across two plants; reproducible pipeline; Streamlit web app delivered |

Challenges and Limitations:

| Limitation | Impact | Future Mitigation |
| --- | --- | --- |
| 34-day dataset only | SARIMA daily (s=7) has limited statistical power; annual/monsoon seasonality cannot be modelled | Multi-year dataset; auto_arima for order selection |
| No external weather forecast input | SARIMA is autoregressive only; cannot anticipate sudden weather changes before they appear in measurements | NWP integration as exogenous variable in SARIMAX |
| LSTM not in Streamlit app | Deep learning too slow for interactive web deployment without GPU | FastAPI backend with pre-trained LSTM; async predictions |
| MAPE on daytime only | Cannot directly compare with papers reporting all-hours MAPE | Document clearly; compute separately for comparison if needed |
| Single geographic location | Results may not generalise to plants in significantly different climates | Test on plants in different MNRE zones; transfer learning |
| CoV fault detection is heuristic | Fixed CoV thresholds may not adapt to seasonal baseline shifts | Replace with Isolation Forest or statistical process control charts |

# Conclusion and Future Work

This project delivers a production-ready solar power forecasting system that meets three simultaneous standards: mathematical rigour (TimeSeriesSplit cross-validation, explicit leakage avoidance for EFFICIENCY and DAILY_YIELD, uncertainty quantification via SARIMA 95% CI), engineering practicality (two-command deployment, interpretable feature importance, automated fault detection), and scientific honesty (model limitations clearly communicated, diagnostic validity tied explicitly to dataset size).

The choice of solar power forecasting at the intersection of renewable energy and Chemical Engineering reflects a genuine industrial need. Indian process industries are increasingly required to integrate renewable energy under net-zero commitments, but lack accessible, open-source forecasting tools that a plant engineer — not a data scientist — can operate and trust. This system closes that gap.

Key contributions: (1) leakage-free feature engineering framework with explicit exclusion of EFFICIENCY and DAILY_YIELD, both documented with root cause; (2) dual-granularity SARIMA with statistically valid full diagnostics on hourly data and simplified residual analysis on daily data; (3) CoV-based fault detection without additional instrumentation; (4) five-model comparison across two plants with consistent architecture and honest uncertainty quantification; (5) a Streamlit web application with hourly/daily SARIMA toggle deployable for free.

## Suggested Future Directions

| Enhancement | Expected Impact | Priority |
| --- | --- | --- |
| auto_arima (pmdarima) | Automatically select optimal (p,d,q,s); removes manual order specification; tested across both plants | High |
| Temporal Fusion Transformer (TFT) | State-of-the-art multi-horizon forecasting with attention-based interpretability; outperforms LSTM on long sequences | High |
| RF + SARIMA ensemble (weighted) | Combine tabular (RF) and temporal (SARIMA) signals; expected 10–20% variance reduction | Medium |
| Multi-year dataset | Enable annual seasonality; LSTM learns summer/monsoon/winter patterns; SARIMA diagnostics more robust | High |
| FastAPI REST endpoint for LSTM | Pre-trained LSTM served via async API; enables SCADA/DCS integration for real-time inverter-level forecasting | High |
| Quantile regression | Probabilistic 15-min predictions at all time horizons; uncertainty bounds beyond SARIMA daily scale | Medium |
| Isolation Forest for fault detection | Replace CoV heuristic with unsupervised anomaly detection; adapts to seasonal baseline automatically | Medium |
| Docker containerisation | One-command deployment across all plants and environments; consistent Python version | Medium |

# References

1. Brownlee, J., 2020. Deep Learning for Time Series Forecasting. Machine Learning Mastery.

2. Pedregosa, F. et al., 2011. Scikit-learn: Machine Learning in Python. Journal of Machine Learning Research, 12, pp.2825–2830.

3. Abadi, M. et al., 2016. TensorFlow: Large-Scale Machine Learning on Heterogeneous Systems. Available at: https://www.tensorflow.org

4. Box, G.E.P., Jenkins, G.M., Reinsel, G.C. and Ljung, G.M., 2015. Time Series Analysis: Forecasting and Control. 5th ed. Wiley.

5. Kaggle, 2020. Solar Power Generation Data. Available at: https://www.kaggle.com/datasets/anikannal/solar-power-generation-data

6. Hochreiter, S. and Schmidhuber, J., 1997. Long Short-Term Memory. Neural Computation, 9(8), pp.1735–1780.

7. Seabold, S. and Perktold, J., 2010. Statsmodels: Econometric and Statistical Modeling with Python. Proceedings of the 9th Python in Science Conference.

8. MNRE, 2023. Annual Report on Renewable Energy in India. Ministry of New and Renewable Energy, Government of India.

9. POSOCO, 2022. Report on Forecasting, Scheduling and Deviation Settlement Mechanism. Power System Operation Corporation of India.

10. Voyant, C. et al., 2017. Machine Learning Methods for Solar Radiation Forecasting: A Review. Renewable Energy, 105, pp.569–582.

# Appendices

## Appendix A — Consistency Checklist (Notebook / App / Report)

| Item | Notebook v4 | Web App | Report v7 |
| --- | --- | --- | --- |
| Time-series model name | SARIMA (SARIMAX) | SARIMA | SARIMA throughout |
| Primary SARIMA granularity | Hourly (s=24, Sec 8) | Hourly (default toggle) | Sections 3.4.2, 6.3 |
| Daily SARIMA alternative | Section 8b | Daily toggle option | Sections 3.4.2, 6.3 table |
| Daily residual analysis | Bar plot + histogram (2 panels) | Bar plot + histogram | Table in Section 3.4.2 |
| Hourly residual diagnostics | plot_diagnostics() all 4 panels | plot_diagnostics() all 4 panels | Table in Section 3.4.2 |
| EFFICIENCY excluded | Yes — commented in code | Yes — not in CANDIDATE list | Section 3.3 table, note |
| DAILY_YIELD excluded | Yes — commented in code | Yes — not in CANDIDATE list | Section 3.3 table, note |
| RF predict fix | rf.predict(X_test) | rf.predict(X_te) | Section 5.1 table |
| LSTM original epochs | 50 epochs (original code) | N/A | Section 3.4.3 |
| LSTM fixed epochs | Max 20 + EarlyStopping(patience=5) | N/A | Sections 3.4.3, 5.1 table |
| MAPE on daytime only | mask = y_true > 0 | mask in compute_metrics() | Sections 4.3, 5.1 table |

## Appendix B — Notebook Structure (53 cells)

- Section 1: Environment Setup — imports, seeds, directories, dark plot theme

- Section 2: Plant Selection (PLANT_ID = 1 or 2 toggle) + Data Pipeline (load, merge, clean, feature engineering with EFFICIENCY and DAILY_YIELD excluded)

- Section 3: EDA — Spearman correlation heatmap, distributions, pairplot

- Section 4: Fault/Abnormality Detection — day-wise profiles, best/worst day comparison, CoV severity classification

- Section 5: Weather Analysis & Seasonal Decomposition (ambient temperature trend, seasonal decomposition, module temperature outlier boxplot)

- Section 6: Inverter Efficiency Analysis — from sensor DC_AC_RATIO and DELTA_TEMPERATURE (not from the excluded EFFICIENCY feature)

- Section 7: ML Models — LR, DT, RF with GridSearchCV + Gini importance + Permutation importance (10 repeats)

- Section 8: SARIMA hourly (s=24, ∼816 pts, full four-panel residual diagnostics) + Section 8b: SARIMA daily (s=7, 34 pts, two-panel residual analysis only)

- Section 9: LSTM (max 20 epochs, EarlyStopping patience=5, Huber loss, sequence length=24)

- Section 10: All-Model Comparison + JSON Report | Section 10b: Plant 1 vs Plant 2 side-by-side comparison

## Appendix C — Generated Output Files

- 01_correlation.png — Spearman correlation heatmap + feature correlation bars with AC_POWER

- 02_distributions.png — Feature distributions for daytime rows only (AC_POWER > 0)

- 03_pairplot.png — Bivariate relationships between AC_POWER, IRRADIATION, temperatures, DELTA_TEMPERATURE

- 04_daywise_dc_power.png — Per-day DC power profiles for all 34 days (fault pattern identification)

- 05_daily_totals.png — Daily AC power totals with best/worst day highlighted in green/red

- 06_best_vs_worst.png — Side-by-side DC power, irradiation, and temperature profiles for best vs worst day

- 07_weather_trends.png — 34-day trends for ambient temperature, module temperature, and irradiation

- 08_seasonal_decomp.png — Trend/seasonality/residual decomposition of ambient temperature (period=7 days)

- 09_module_temp_boxes.png — Module temperature box plots by day (outliers flag thermal fault events)

- 10_efficiency.png — DC_AC_RATIO vs output curve and DELTA_TEMPERATURE vs DC_POWER scatter (sensor-based, not EFFICIENCY feature)

- 11_feature_importance.png — Gini importance (left) and Permutation importance ±std (right) side-by-side

- 12_rf_evaluation.png — RF actual vs predicted scatter + residual plot

- 13_sarima_split.png — Hourly train/test split visualisation

- 14_sarima_forecast.png — SARIMA hourly forecast vs actual with 95% confidence interval band

- 15_sarima_diagnostics.png — Full four-panel residual diagnostics (hourly model only; all panels statistically valid at n≈653)

- 16_lstm_evaluation.png — LSTM training/validation loss curves, time-series overlay, scatter, error distribution

- 17_final_comparison.png — All ML models side-by-side MAE/RMSE/R² comparison bar charts

- metrics_report.json — Structured metrics with leakage notes, scale notes, best model selection, SARIMA configuration used

# Auxiliaries

Please add the below-mentioned links.

Web link:

https://solarpowerforecasting-7fmabm2xo5dkudkycnpdwu.streamlit.app/

GitHub Repo:

https://github.com/chauhan22adarsh/Solar_Power_Forecasting

Data Source:

https://www.kaggle.com/datasets/anikannal/solar-power-generation-data

Required files: Plant_1_Generation_Data.csv, Plant_1_Weather_Sensor_Data.csv, Plant_2_Generation_Data.csv, Plant_2_Weather_Sensor_Data.csv

Python file:

https://github.com/chauhan22adarsh/Solar_Power_Forecasting/blob/main/app.py